;USKI-5 PROGRAM(C)PROVIDES DELAYS FOR QSK CW OPERATION WITH AN EXTERNAL KEYING DEVICE BUT NORMAL(NON-SEQUENCED)LOCAL PTT/VOX CONTROL FROM XCVR.

;WARNING: THIS PROGRAM IS NOT FOR USE WITH TMAs, TRANSVERTERS, ETC., THAT REQUIRE DELAYED SEQUENCING TO PREVENT DAMAGE. DELAYS APPLY TO CW OPERATION ONLY!

;REFER TO THE USKI-5 USER MANUAL FOR PROPER INPUT AND OUTPUT CONNECTIONS TO THE SEQUENCER

;ALL DELAYS ARE IN MILLISECONDS (ms)

symbol CW_START_DEL_1 = 8 * 0			;output 1 CW start delay

symbol CW_START_DEL_2 = 8 * 0			;output 2 CW start delay

symbol CW_START_DEL_3 = 8 * 0			;output 3 CW start delay

symbol CW_START_DEL_4 = 8 * 0			;output 4 CW start delay

symbol CW_START_DEL_5 = 8 * 20		;output 5 CW start delay

symbol CW_END_DEL_5 = 8 * 20			;output 5 CW end delay, usually sum of START_DEL_1, 2, 3, 4, AND 5

symbol CW_END_DEL_4 = 8 * 0			;output 4 CW end delay

symbol CW_END_DEL_3 = 8 * 0			;output 3 CW end delay

symbol CW_END_DEL_2 = 8 * 0			;output 2 CW end delay

symbol CW_END_DEL_1 = 8 * 0			;output 1 CW end delay

pullup on						;turns on pullup resistors on inputs

setfreq M32						;overclock to 32 MHz

main:

initialize:						;initialize outputs upon power up. low = OPEN, high = CLOSED

low 1

low 2

low 3

low 4

low 5

standby:						;standby loop

setint %00000000, %00000100		;sets interrupt on C.2 (J2R)

if pinC.2 = 0 then goto standby	;checks for interrupt on C.2 (J2R)

bit0 = 0					;clears interrupt flag if necessary

if pinC.1 = 0 then goto cwstart	;if J1T closes, goes to cwstart

if pinC.3 = 0 then goto faststart	;if J1R closes, goes to fast start routine

goto standby

cwhold:						;holds program here after each CW symbol waiting for next symbol or until send opens (in case of semi-breakin operation)

setint %00000000, %00000100		;sets interrupt on C.2 (J2R)

if bit0 = 1 then goto cwend5		;checks for interrupt

if pinC.1 = 0 then goto cwstart	;if J1T closes, go to cwstart

if pinC.3 = 0 then goto cwhold	;if J2T still closed, loop back to cwhold

goto standby				;return to standby

cwstart:						;starts cw tx procedure here

pause CW_START_DEL_1			;output 1 CW start delay

if bit0 = 1 then goto standby

high 1					;switches output 1

pause	CW_START_DEL_2			;output 2 CW start delay

if bit0 = 1 then goto cwend1

high 2					;switches output 2

pause CW_START_DEL_3			;output 3 CW start delay

if bit0 = 1 then goto cwend2

high 3					;switches output 3

pause CW_START_DEL_4			;output 4 CW start delay

if bit0 = 1 then goto cwend3

high 4					;switches output 4

pause CW_START_DEL_5			;output 5 CW start delay

if bit0 = 1 then goto cwend4

high 5					;switches output 5

cwend:

if bit0 = 1 then goto cwend5		;checks for interrupt while waiting for cw keyup

if pinC.1 = 0 then goto cwend		;waits here until CW keyup

cwend5:

bit0 = 0

pause CW_END_DEL_5			;output 5 CW end delay

low 5						;unswitches output 5

pause CW_END_DEL_4			;output 4 CW end delay

cwend4:

low 4						;unswitches output 4

pause CW_END_DEL_3			;output 3 CW end delay

cwend3:

low 3						;unswitches output 3

pause CW_END_DEL_2			;output 2 CW end delay

cwend2:

low 2						;unswitches output 2

pause CW_END_DEL_1			;output 1 CW end delay

cwend1:

low 1						;unswitches output 1

goto cwhold					;go to cw hold

faststart:						;starts fast tx procedure here with no delays

setint %00000000,%00000100

high 1					;switches output 1

high 2					;switches output 2

high 3					;switches output 3

high 4					;switches output 4

;output 5 not used in fast start routine

ptthold:

if bit0 = 1 then goto fastend

if pinC.3 = 0 then goto ptthold	;waits intil fast ptt ends

fastend:

;output 5 not used in fast start routine

low 4						;unswitches output 4

low 3						;unswitches output 3

low 2						;unswitches output 2

low 1						;unswitches output 1

goto standby				;return to standby

interrupt:						;interrupt routine

bit0 = 1					;set interrupt flag on

return					;return to point where interrupt occurred